Table of Contents

Introduction . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2           Quick Install . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3    Quick Set Up . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3       Quick Download . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7    Quick Processing . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8 Warranty Information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9  Download Data  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 10   Contiguous Dump  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 10 Dump Since Set  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  11   Entire Memory   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .12 Data to PC   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13                Set DataLogger   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .14              Auto Set  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14    Reset Auto Set . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .15 Current Status  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  18        View DataBank  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19               Erase DataBank  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .20     Recycle Bin  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .21          View Plots  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  22       Name & Message . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  24          New Batteries . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  25         View Status  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  26         Print File . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  27 Starting the START MENU . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  28 The Integrated Water Level Data Logger System . . . . . . . . . . . . . . . .  29   PC Transfer Software . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  33 Loading the HP with the Start Library using Kermit . . . . . . 35               User Well Pipe Construction . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  37        Rain Gauge Data Logger with Self-Emptying Tipping Bucket . . . . . . . . . . 38 Pressure Water Level Data Logger . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 39 Specifications . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

- Introduction

This manual covers the Infinities USA, Inc. Data Logger Series Electronic Water Level Data Logger and the Rain Gauge Data Logger with Self-Emptying Tipping Bucket.  The operation of the Water Level Data Logger and the Rain Gauge Data Logger are similar. The functions of the Data Loggers are explained using the Ultrasonic Water Level Data Logger. Additional notes pertaining to the Rain Gauge Data Logger or the Pressure Water Level Data Logger are covered separately toward the end of this manual. Any questions about the Data Logger System can be directed to our toll-free line at  (888) 808-5488.

The water level Data Logger consists of three main components: 1) the Data Logger with screened pipe, 2) the optional Hewlett Packard 48GX pocket calculator /computer with serial cable, and 3) the Data Logger Series PC Transfer Software for the PC. The primary function of each component is described below.

- The water level Data Logger measures and records, at programmed intervals, the water level relative to the zero mark of the 1 ¼” to 3” screened PVC pipe.

- The Zero Mark is normally 24” to 36” below the ultrasonic sensor on the bottom of the Data Logger unit.

- A user offset is provided allowing the user to set the zero mark from 0 to 255 inches below the pipe zero mark. This gives the user flexibility in inserting in the ground only the length of pipe necessary for the interested range of water levels.

- An elevation feature allows the user to effectively set the Zero Mark from 0 to 655.35”. Since the Zero Mark is normally 24” to 36” down from the top of the PVC pipe, the elevation reflects the setting of 24” to 36”.

- The optional HP48GX pocket calculator/computer provides the user a handheld interface to the Data Logger.

- Set up information is entered into the Data Logger using the HP via serial cable.

- Data logged by the Data Logger is downloaded using the HP.

- Data downloaded by the HP from the Data Logger is transferred to a PC via serial cable.

- The Data Logger Series PC Transfer Software runs on a PC enabling transfer of information and data between a PC and a Data Logger, via serial cable and null modem adapter. The software processes the information into a directory or file folder on the hard drive of a PC. The software is also capable of transferring data from a HP48 to a PC.

Introduction

- Set-up information can be entered into the Data Logger using a PC via PC serial cable and null modem adapter. Data Loggers can be downloaded using a PC. Data files can be printed.

- Data downloaded by the HP from the Data Logger can be transferred to a PC via HP serial cable.

Quick Start

Infinities USA, Inc. strives to ensure our instruments are easy-to-use, from A) installation and B) set up, to C) downloading and D) processing data. The following brief introduction is intended to help the user to quickly become acquainted with the Data Logger Series System. All functions of the Data Logger are covered in detail in this manual and can be found by using the Table of Contents.

A) Quick Install

Each Data Logger is shipped without the batteries installed. The batteries are included. To install batteries, see page 25.

To install in the ground:

- bore a hole for the size of well pipe,

- insert the well pipe to the zero mark (or employ user offset, page 5),

make plumb, backfill with coarse dirt or sand, tamp down, leaving 6 inches for a seal,

3) seal with Bentonite pellets and/or a mound of 50/50 mixture of Bentonite/soil mixture to prevent surface water from entering,

4) place the Data Logger on the pipe until the top of the pipe seats in the PVC                                         fitting on the bottom of the Data Logger.

- Quick Set Up

Note: To set up the Data Logger using the PC, see page 33.

- Setting up the Data Logger requires I) turning the HP48GX on, II) connecting the HP48GX to the Data Logger and III) using the menu to set the offset, time to begin reading, and reading interval.

- I) Turning the HP48GX on

- Press and release the ON key - lower left key on HP48GX keyboard. The Datalogger Start Menu is displayed with Download Data highlighted. Pressing the ON key twice “cancels” the DataLogger Start Menu. See page 28 on how to display the DataLogger Start Menu if the screen below does not appear.

Throughout this manual, when instructed to press a key, the instruction is intended for the user to press and release the key as when touch-typing on a keyboard, not to press the key and continue to hold the key down.

Introduction

- II) Connecting the Data Logger to the HP with the Serial Cable

- Remove Data Logger Cap.

- With the HP48 on, connect the 4-pin HP connector on the HP serial cable to the top end of the HP. The HP connector should a) face HP logo upward, b) snap in, and c) not fit flush to the HP. Cables without the HP logo should be inserted with the flat part of the 4-pin connector facing upward on the HP.

- Connect the 9-pin connector on the HP serial cable to the Data Logger.

- III) The HP48 User Interface

- The HP48 User Interface is a menu-driven system providing full access to all functions of the Data Logger to the user. By using the cursor keys (arrow keys in inverted T configuration: up, down, left, right beside the letters K, Q, P, and R, respectively) on the HP, the user can choose a function by highlighting a desired menu item. Pressing and releasing the large ENTER key on the HP executes the highlighted item.

- **Important:** Be sure the current time and date are set on the HP. Use the green-shift "Time" key (the green -shift 4 key) to "Set time, date…" on the HP.

- Use the Down Arrow cursor key (the Q key) to arrow down to Set DataLogger.

- Press ENTER with Set DataLogger highlighted.

- Use the Arrow keys ( the K, P, Q, R keys) to highlight Reset Auto Set.

- Press ENTER with Reset Auto Set highlighted and the OFFSET screen appears.

- Enter the number of inches from the Data Logger Zero Mark down to the user Zero Mark (normally the place on the pipe at ground level) in whole inches up to 255 inches of offset. Use a 0 offset for Pressure Water Level Data Loggers. Use the Elevation feature to set the elevation of the brass sensor pickup. For example, for the brass sensor pickup 50 inches below the ground surface, use an elevation of -50. On the HP, enter 50 followed by the +/- key.

Introduction

- Here is an example of a 15-inch offset being entered.

- Enter the time for the Data Logger to log the first reading. The screen displays a Days, Hours, Minutes, and Seconds field with defaults of zero.

- Enter the number of days until the first reading will be recorded.

- Enter the number of the hour of the first reading and press OK. For example, for 8pm enter 20 in the Hours field. For 8am, enter 8.

- Enter the number of the minute of the first reading and press OK. For 8pm, enter 20 in the Hours field, and press OK with 0 in the Minutes field.

- Enter the number of the second of the first reading and press OK.

- When the Day, Hour, Minute, and Second fields have the time of the first reading in them, press OK to go to the next display.

- Time Interval Between Readings displays an Hours field, a Minutes field, and a Seconds field with defaults of zero.

- Enter the interval between readings, pressing OK after setting each field, and by pressing OK when all fields are set. If an interval of 1 hour is desired, enter 1 in the Hours field by pressing 1 and ENTER, followed by ENTER again which enters the default 0 for the minutes and the default 0 for seconds. The Data Logger will now be set to 24 readings per day.

More on Setting the Time to Begin

Setting the Time to Begin has built into it the following features:

- The Time to Begin will always be programmed by the HP to occur in the future. If the user attempts to set the Time to Begin for 08:00am at an interval of one reading per hour, and the time is 11:30am, the software will detect that 08:00am has passed and will set the Time to Begin to the next available time in the future of 11:30am using the 1 hour interval from 08:00am. In this example, the Time to Begin will occur at 12:00 noon with an interval of one reading per hour.

Introduction

- The offset is sent, along with Time of the HP, Time to Begin, and Interval information to the Data Logger (in this example, the Ultrasonic Data Logger will log water levels 15 inches down from the Data Logger zero mark as zero inches). The HP receives and stores the Data Logger data and displays information about the Data Logger in the next three display screens.

- The user name

- Time Data Logger was downloaded

- Time of next reading

- Number of Readings per Day

- The Water Level (negative denotes below the zero mark)

- The inches of offset (distance between the ultrasonic device zero mark and user zero mark)

- User prompt

- Serial Number

- Number of measurements in the download file

- Battery Voltage, Battery Capacity, Battery Installation Date, Temperature

- User Message (31 characters max.)

- Time and Date of Memory Wrap

- User Prompt

- Available memory free for downloads

- Memory used by DataBank files

- Memory used by Recycle files

- Elevation setting of the Data Logger

- Latitude and Longitude settings of the Data Logger

Introduction

C) Quick Download

- Connect the 9-pin end of the serial cable to the Data Logger and the other end of the cable (the 4-pin connector) to the top end of the HP48. Orient the HP logo on the cable connector to face upward.

- From the Start Menu, highlight Download Data and press ENTER.

- Contiguous Dump is highlighted. Press ENTER.

- Contiguous Dump provides the user with data that has been recorded since the last Contiguous Dump. As in Quick Set Up, three information screens give Data Logger status. See pages 11 and 12 for the other types of downloads, Dump Since Set and Entire Memory.

- Replace the cap by screwing the cap back on until tight. Add or replace Teflon tape on threads as necessary to maintain a water-tight/air-tight seal.

Introduction

- Quick Processing

- PC. To transfer data to the PC, first install Data Logger Series Transfer Software. Insert Disk1 in a: drive or b: drive, select Run on the Start Menu of the PC and type a:\setup or b:\setup and follow instructions on the PC display.

- When the software is installed on the PC, connect the cable from the HP to an unused COM port: 1, 2, 3, or 4, on the PC.

- PC. On the PC use Start, Programs, PC Transfer 95/98/NT and the program displays a menu. Select Handheld Retrieval Device. Select the COM port to which the cable is connected.

- PC. Click on the Receive Files button.

- HP. On the HP, arrow down to Data to PC and press OK.

- HP. The next menu reminds the user to Run PC Software. Press ENTER.

- PC. File transfer has finished when the PC display shows the message “Finished Processing Files”.

- PC. Use File, Open, and select a file to view or print.

- PC. The downloaded files are stored on the PC hard drive in the Infilog directory. Import the downloaded data files that are ASCII text files, comma delimited, into graphing programs to chart or graph.

- When using Excel, use File, Open, in Excel, select Files of All Types, select the date named subdirectory and choose the data file you want to open. The text import wizard (1 of 3) will allow you to select a data type that is delimited. Select a comma delimiter (2 of 3), because commas delimit the data file. Finish the text import wizard and the data file should be in columns in the spreadsheet. To produce a chart, highlight the data in the two rightmost columns and select xy scatter plot. Edit the chart. Right-click on the x-axis to format the number type to date format.

For more information on transferring data to the PC, see Data to PC, page 13.

For further information on the Data Logger System, see Table of Contents.

Warranty Information

Warranty

Infinities USA, Inc. warrants its products against defects in materials and workmanship for a period of 1 year.

This warranty does not apply if the product has been modified or damaged by accident, abuse, or misuse.

Copyrights and Trademarks

Copyright 1999 by Infinities USA, Inc. All rights reserved. Infinities USA, Inc. and all variations of the Infinities logo are trademarks of Infinities USA, Inc. HP is a registered trademark of Hewlett Packard. Other brand and product names are trademarks or registered trademarks of their respective holders.

Disclaimer of Liability

Infinities USA, Inc. is not responsible for special, incidental, or consequential damages resulting from any breach of warranty, or under any legal theory, including lost profits, downtime, goodwill, damage to or replacement of equipment or property, and any costs or recovering, reprogramming, or reproducing any data stored in or used with Infinities USA, Inc. products.

Download Data

Contiguous Dump

Start Menu

Select Type of Download

Contiguous Dump is used to download data from the Data Logger to the HP via the serial cable. The Contiguous Dump will download data from the last time data was downloaded using Contiguous Dump.

For example, a Data Logger is set to begin reading on April 12, 1996 at 10 AM, at an interval of one reading per hour. The Data Logger is read using a Contiguous Dump. Now when the user returns on June 23, 1996 at 2 PM and uses a Contiguous Dump, the data file contains data from April 12, 1996 to June 23, 1996. On September 7, 1996, the user returns to the field and uses a Contiguous Dump. The data file downloaded by the HP contains data from June 23, 1996 to September 7, 1996.

If the user had used a Dump Since Set on September 7, 1996 instead of a Contiguous Dump, the data file downloaded by the HP would contain data from April 12, 1996 to September 7, 1996.

Wrap Around Memory

The Data Logger records 3906 readings and continues to log data by writing over the oldest reading in the memory. As an aid to the user, each data file downloaded to the HP contains the date and time that the oldest reading will be overwritten.  The date and time are located in the second information screen after a download.

Wrap Around Date and Time

When Wrap Around occurs, and a Contiguous Dump is performed, the data file will contain all of the data in memory, up to 3906 readings, of the latest readings.

Download Data

Dump Since Set

Select Type of Download

Dump Since Set is used to download data from the Data Logger to the HP via the serial cable. Dump Since Set will download data from the time the user set the Data Logger using Set DataLogger. This method gives the user all of the data in the data set no matter how many Contiguous Dumps are performed.

For example, a Data Logger is set to begin reading on December 11, 1996 with an interval of 2 readings per day. The user can perform Contiguous Dumps each month. On October 3, 1998, the user can perform a Dump Since Set and the data file will contain data from December 11, 1996 to October 3, 1998.

A Data Logger is Set using Set DataLogger, Auto Set or Reset Auto Set.

Memory Wrap

As discussed on the previous page, data is written on top of the oldest reading in memory after the memory has stored more than 3906 readings. Using Dump Since Set after more than 3906 readings have been recorded by the Data Logger generates a data file with up to 3906 of the latest readings.

To aid the user, each data file contains the wrap around date and time.

Wrap Around Date and Time

Download Data

Entire Memory

Select Type of Download

Entire Memory is used to download data from the Data Logger to the HP via the serial cable. Entire Memory will download all data from all data sets.

For example, a user sets a Data Logger using Auto Set or Reset Auto Set for one reading per hour and records a month of readings. The user then sets the Data Logger for one reading per day and records readings for a month. The user performs Download Data, Entire Memory. The downloaded data file includes both data sets and any other data sets the Data Logger may have logged prior to the most recent two sets. Using Dump Since Set will download only the one reading per day data set, in this case.

Entire Memory dumps all data in the Data Logger.

Data to PC

Data to PC is used to download data from the HP to the PC. Pressing OK at the Data to PC menu displays a second menu to remind the user to Run PC Software.  The Data Logger Series Transfer Software for the PC provides a quick and easy way to transfer data from the HP to a PC.

To connect the HP to the PC:

- Connect the PC end of the HP cable to the serial port on the computer.

- Keep the HP logo on the 4-pin connector facing up, then plug the HP cable into the HP48. The plug should lightly snap into place, not quite flush with the HP.

To transfer files from HP to PC:

- PC. Run the Data Logger Series PC Transfer Software. If the Data Logger Series PC Transfer Software is not installed, from Run on the PC, type A:\setup or B:\setup and follow instructions on the PC display. The Software will create a shortcut item on the PC Start Menu under Programs. Click on Start, Programs, and PC Transfer 95 98 NT to run.

- When the software is installed on the PC, connect the cable from the HP to an unused Com port: 1, 2, 3, or 4, on the PC.

- With the PC Transfer 95 98 NT software running, select Handheld Retrieval Device. Select the Com port to which the cable is connected and click on the Receive Files button. When the Com port selection box dims along with the Receive Files button, the PC is ready to receive files from the HP.

- HP.  From the HP Start Menu shown above, arrow down to Data to PC and press the button associated with the OK at the bottom right of the HP display.

- HP. With the PC software running and the PC displaying the reminder, Run PC Software, press the HP button associated with the OK on the HP display.

The files are transferred and processed by the PC. When the PC displays “Processing Finished”, processed files are now located in the InfiLog directory or folder. Use File, Open, to view or print the processed files.

Set DataLogger

Auto Set

Auto Set is useful when setting up multiple Data Loggers to take measurements at the same interval and time as the device set up using Reset Auto Set.

Note: We recommend using an offset of 0, and use the Elevation feature to set the elevation of the sensor. The brass pickup on the pressure sensor is below the ground surface for measuring ground water levels. For a sensor 50 inches below the surface, use a 0 offset and a -50 elevation. On the HP use the +/- key to toggle the sign.

Auto Set prompts the user for the offset, using the offset entered from the previous Set DataLogger. Enter the number of inches from the Data Logger zero mark down to the user zero mark in whole inches up to 255 inches of offset.

Here is a 15-inch offset being entered. Press ENTER or OK and the zero is replaced by the 15.

Press ENTER or the OK button and the offset of 15 inches is sent to the Data Logger, which will now log water levels 15 inches down from the Data Logger zero mark as zero.

Here the default offset is zero inches down from the Data Logger zero mark. Pressing the OK button here uses the default 0 inches shown above. Water at the Data Logger zero level will be stored as zero.

Set DataLogger

Reset Auto Set

Set DataLogger

Reset Auto Set is used to set the Data Logger with the HP via the HP serial cable.

- Connect the HP cable from the HP to the Data Logger.

- From the Start Menu, arrow down to Set DataLogger and press OK.

- Arrow down in the second menu to Reset Auto Set and press OK.

- When prompted for inches of offset, enter the number (no decimal, whole number only) of inches from the Data Logger zero mark to the user zero mark. Valid offsets are from 0 to 255. Note: With the addition of the Elevation feature, a 0 offset can be used, and elevation can be used to tell the data logger where the sensor is relative to a natural zero like the ground surface. For example, for the ultrasonic sensor 36 inches above the ground surface, and elevation of 36 should be used. For the pressure sensor that is, for example, 50 inches below the ground surface, use a -50 inch elevation.

- Here is an offset of 15 inches being entered.

- Pressing OK inserts 15 into the offset field.

Set DataLogger

Reset Auto Set

- Time to Begin displays a Days, Hours,  Minutes, and Seconds field with defaults of zero.

Enter the number of the Days until the Time to Begin recording the first reading.

Enter the number of the hour of the Time to Begin and press OK.

Enter the number of the minute of the Time to Begin and press OK.

Enter the number of the second of the Time to Begin and press OK.

When the Days, Hours, Minutes, and Seconds fields, have the Time to Begin in them, press OK to go to the next display.

- Time Interval Between Readings displays an Hours field, a Minutes field, and a Seconds field with defaults of 0.

Enter the interval between readings, pressing OK after setting each field, and by pressing OK when all fields are set. If an interval of 24 hours is desired, enter 24 in the Hours field, press OK, followed by OK again, and the Data Logger is set to a 1 reading per day.

More on Time to Begin

Time to Begin has built into it the following features:

- The Time to Begin will always occur in the future. If the user attempts to set an the Time to Begin for 8:00am at an interval of one reading per hour, and the time is 5:00pm, the software will detect that the 8:00am time has passed and will set the Time to Begin for 6:00pm that day with an interval of one reading per hour.

- If the time is 12:00 noon and the user wants to begin recordings at 8:00pm, with an interval of one reading per hour, enter in the Hours field of Time to Begin the number 20, which is the number of hours in the 24-hour format for 8:00pm.

Set DataLogger

Reset Auto Set

- When the display shows Time to Begin with defaults of zero for the Hour, Minute, and Second, pressing OK will alert the software to automatically choose the time in the future based on the Time Interval Between Readings. For example, if the user presses OK when the display shows Time to Begin with defaults of zero for the Hour, Minute, and Second, the display then prompts for Time Interval Between Readings. If the user enters 6 in the Hours field and presses OK, followed by OK to bypass the defaulted zero in the Minutes field, the software computes a Time to Begin using the current time and the interval. If the current time were 10:00am, then a Time to Begin would be 12:00 noon and readings would occur every 6 hours thereafter.

- After setting the Time to Begin and Time Interval Between Readings, the HP reads the Data Logger and displays the Time to Begin and Time Interval Between Readings. The user should verify these settings.

Current Status

Current Status returns the current state of the Data Logger via the cable to the HP.

The first screen on the display shows the following information:

- The user name

- The time the Data Logger was downloaded

- Time of next reading

- Number of Readings per Day

- The Water Level (negative denotes below zero mark)

- The inches of offset (distance between device zero mark and user zero mark)

- User prompt

- Serial Number

- Number of measurements in download file

- Battery Voltage, Battery Capacity, Battery Installation Date, Temperature at time of download

- User Message (31 characters Max.)

- Time and Date of Memory Wrap

- User Prompt

- Available memory free for downloads

- Memory used by DataBank files

- Memory used by Recycle files

- Elevation setting of the Data Logger

- Latitude and Longitude settings of the Data Logger

View DataBank

View DataBank allows user to view any file downloaded into the HP.

- Arrow down to View DataBank in Start Menu and press OK.

- Choose DataBank file using arrow keys and press OK.

- Press Cont to view.

- Press Cont to view each measurement recorded until “End of Data” is reached. "Out of Range" is recorded when there is no reflection received from the water. "Out of Range" can occur when the water level is more than 35 feet for Model #420. For Model #220, "Out of Range" can occur when the water level is above or below the measurable range in the monitoring well.

- Pressing KILL exits the View Files function.

- Press START for Start Menu.

Erase DataBank

Erase DataBank erases DataBank files in the Dat directory or folder on the HP, placing copies of the files in the Recycle directory. Erasing the DataBank empties the Dat directory and retains the files in the Recyc directory. This emptying of the Dat directory readies the HP for downloading a group of Data Logger files. These files can then be downloaded to the PC and located in their own directory. The Recycle files, when downloaded to the PC, will be place in a separate directory. To erase the Recycle files, see page 21.

- Arrow down to Erase DataBank in the Start Menu.

- Press OK.

- Files are transferred to the Recycle Bin when “Empty DataBank” is displayed on the HP display.

Recycle Bin

Recycle Bin performs similar functions on files in the Recycle directory as similarly named functions in the Start Menu perform on files in the Dat directory. The Recycle Bin contains all files erased using Erase DataBank.

Features of the Recycle Bin:

- When using View Files, the file ACC.1 in the Dat directory may be renamed ACC.17 when erased from the Dat directory, depending on the number of files in the Recycle directory.

- All files erased from the Dat directory can be transferred to the PC using Files to PC.

- Erase Recycle removes all Data Logger files in Recycle Bin or directory.

- View Plots allows the user to plot a selected file in the Recycle directory to the display.

- View Status gives the user access to the file information such as time of download, water level, memory wrap time and date, etc.

View Plots

View Plots allows user to see a plot of any data file stored in the HP from the Data Logger. Press OK.

Use arrow keys to arrow down or up and press OK to select a file to plot to the display. Use the CONT white key or the purple-shift ON key to continue between pages of data if HALT appears at the top of the screen.

The plot function plots a page of Data Logger memory.

The (X,Y) key can be pressed to see the water level on various parts of the display. By driving the cursor with the arrow keys, the water level is displayed.

Here the Y value is -13.05 inches. The X value plotted can be decoded by pressing the ENTER key. When the ENTER key is pressed, a snapshot of the cursor position is stored.

To display the snapshot press the upper right F key to exit the coordinate mode.

Then press CANCL to exit the plot screen.

View Plots

The snapshot gives the time and date, water level, and offset.

Pressing CANCL returns to the next memory page and plots it. The user can make more snapshots and view them as above.

Driving the cursor and pressing ENTER stores as many snapshots as the user may want.

Pressing CANCL gives the user the information from the snapshots.

To exit from the View Plots function before all data is viewed, the user must press the ON key to CANCEL from the plotting program. Then press START for the Start Menu.

Name & Message

Name & Message give the user the ability to name each Data Logger instrument and store a 31 character message in each Data Logger.

The User Name can be a maximum of eight characters beginning with a letter and the remainder of the User Name consisting of letters or numbers. Do not use underscore characters in the User Name. Names with underscores cannot be stored by the HP. To enter letters, make sure the alpha annunciator is on at the very top of the display pressing the alpha key to the left of the 7 key. Use the arrow keys to drive the cursor left or right. Use the back-arrow key to backspace. Use the DEL key to delete a character.

Again, be sure alpha is on to type letters. To access lower-case letters, with alpha on, press the purple left-shift key followed by the key whose letter is desired.

Do not use the arrow keys with alpha on or the letters K, P, Q, R will go on the display. Toggle alpha off to use the arrow keys.

New Batteries

Normal battery life for the Data Logger is four years. Battery life depends on various factors such as frequent extreme temperature variations to frequency of water level measurement. Measurements as frequent as one measurement per second use little battery power over the 3906-measurement memory. However, allowing the Data Logger to continue for days at one measurement per second will lower battery life. Moreover, battery life for a Data Logger set for fifteen-minute intervals shows similar battery life for a Data Logger set for twenty-four hour intervals. Battery condition is monitored on-board the Data Logger. The battery voltage and the percentage of battery power remaining are included in every data file that is downloaded or can be viewed using Current Status.

- To replace batteries, remove battery holder covers. With a key, pen, or screwdriver, insert into one end of battery holder cover and tilt the battery holder cover latch outward to unlatch as you lift the end of the battery holder cover. Unlatch one or both ends and remove battery holder cover.

- Repeat for remaining three battery holder covers.

- Remove all batteries. Using a fingertip on the positive end or top end of the battery works best.

- Replace with four AA alkaline batteries taking care not to get oils from your hands on the metal ends of the batteries. The battery polarities are marked on the battery holders. A battery is easier to install if the negative end is inserted first, then the positive end is snapped into place.

- Replace battery holder covers, ensuring the ends are latched to hold batteries securely.

- Set the battery installation date in the Data Logger by arrowing down to New Batteries and press ENTER or OK on the HP48.

- Select “Yes” when prompted to WRITE NEW BATTERY DATE. The new month/year are written into the Data Logger. A Current Status is performed to verify the battery installation date.

- When using the PC Transfer Software, under the Utilities section of Set DataLogger, choose New Batteries and select OK when prompted to write the new month/year.

- Reset the Data Logger with the HP48 by running Set DataLogger, Autoset, or Reset Autoset. Use Set DataLogger if using a PC. The current time and date, time of next measurement, and interval will be sent to the DataLogger.

View Status

View Status allows the user to view the status of any file downloaded from a Data Logger instrument.

- Arrow down from the Start Menu to View Status or Arrow up for a shortcut to View Status.

- Press OK.

- Select file to view by using arrow down and press OK.

- Three screens of information follow, the same as Current Status or when downloading files.

Press any key for the next screen.

Press any key for the next screen.

Press any key for the Start Menu.

Print File

The Print File command on the HP48 menu is used in conjunction with the HP 82240B Infrared Printer.

- Turn on Infrared Printer.

- Arrow down on HP48 to Print File or arrow up for a shortcut to Print File. Press OK or ENTER.

- Highlight file to print using arrow keys, align HP48 in front of Infrared Printer, and begin printing by pressing OK or ENTER.

- The HP48 will display and send the file selected to the Infrared Printer. The HP48 will continue to send data to the Infrared Printer until END OF DATA.

- Turn off the Infrared Printer when finished printing.

### Starting the START MENU

All functions of the Data Logger are performed via the Start Menu of the HP48GX. The following explains 1) where the Start Menu is located and 2) how to start the Start Menu.

The Start Menu gives the user access to all functions of the Data Logger. The Start Menu can be exited to use the HP48GX for computations, etc. To get back to the Start Menu, located in the Infinities Directory:

- Halt the calculator by pressing the ON button twice. This cancels any HP screen you may be in.

- Press the green shift key followed by the HOME key (3rd key down on the left). This puts the calculator in the HOME directory or root directory.

- Press the CST key (2nd row, 3rd key over from the left). This is the CuSTom menu in the HOME directory. Start Library, STLIB, is in the lower left of the screen. Press the top, left white key under STLIB for the Start Library.

- Press the white key directly under the START on the bottom of the screen. This starts the START MENU.

The user may create other directories and create CuSTom menus and use the HP48G+ for other tasks besides functioning with the Data Logger. To get back to the START MENU of the Data Logger, follow the four steps above.

#### The Integrated Water Level Data Logger System

The Infinities Water Level Data Logger System is integrated to provide ease of installation and ease of use.

The System includes:

- The Water Level Data Logger.

- The optional HP48G+ programmed with the START MENU library.

- A HP serial cable to allow the HP to communicate both with the Data Logger and a PC.

- PC Transfer Software for the PC for communicating with the Data Logger (use null modem adapter) and to facilitate file transfers from the HP to the PC. (Do not use null modem adapter when using HP cable.)

- For measuring groundwater, PVC pipe with 0.008 in slots to keep dirt out but let water flow in and out freely.

- This manual.

System Overview

- The 1 ¼” to 3” PVC pipe is placed in the water whose level is to be recorded over time. Slots keep dirt out allowing the fluctuating water surface to be measured up to the 20-foot length of the pipe.

- The Water Level Data Logger, with integrated ultrasonic transducer to sense the range of the water level, is placed on the pipe. The Data Logger should seat against the top of the pipe.

- Removing the waterproof screw-on/screw-off cap exposes the 9-pin connector. The waterproof cap also protects the battery compartment, with its four AA alkaline batteries.

- The HP serial cable connects the HP, using the 4-pin connector, to the Data Logger, using the 9-pin connector. The rounded shape of the bottom of the connector on the HP should match the shape of the rounded bottom of the 4-pin connector on the HP cable. The 4-pin connector snaps in and does not fit flush.

- The HP is programmed with the START MENU library which performs all data gathering functions, from a) setup and downloading, to b) on screen analysis or printing to the field printer (optional), or c) transferring the data files to a PC for extensive analysis and long-term storage.

- This manual begins with an Introduction to the Water Level System. Users can quickly get started using the Introduction. Each START MENU item has its own section in the manual for further information. This section at the end of the manual describes the details of the Water Level System.

The Integrated Water Level Data Logger System

Water Level System Features

The Water Level Data Logger

Batteries. Four standard AA alkaline batteries (included) power the Data Logger. The New Batteries section of the manual, page 25, describes battery life, battery replacement, and the on-board battery gauge of the Data Logger.

Memory. The Data Logger has a memory capacity of 3906 water level recordings. Data is not lost in the absence of battery power. Data is retained for greater than 200 years with no power.

Downloading data features three methods: 1) Contiguous Dump, 2) Dump Since Set, and 3) Entire Memory. These methods help the user get the data desired.

Contiguous Dump provides the user with data since the user’s last download with Contiguous Dump. Using Contiguous Dump, in effect, places a bookmark in the Data Logger. The bookmark can move each time the Data Logger is downloaded using Contiguous Dump.

Dump Since Set gives the user the data file that contains all of the data since the Data Logger was set with the offset, the Time to Begin, and interval. This data file gives the user all of the data since the Data Logger was Set. Using Dump Since Set places a different bookmark in memory, one that does not move until the Data Logger is Set again.

Entire Memory gives the user all of the data in the Data Logger, including all data sets in memory. A data set in memory is created using Set DataLogger.

Data is downloaded from the Data Logger to the HP for analysis and PC Transfer yet remains stored safely in the Data Logger in case the HP is lost or damaged in transport. If allowed to record more than 3906 measurements, the Data Logger overwrites the oldest reading with the 3907th measurement. Each data file that is downloaded displays the time and date of this memory wrap-around. This wrap-around date and time allows the user to plan to download the Data Logger before the oldest reading is written over with new data.

Name and Message. Each DataLogger has a unique serial number, such as N3370B68. Each Data Logger can be named using up to 8 characters beginning with a letter. This user name will be used in every data file as well as being the data file name when the file is transferred to a PC. This feature gives the user the ability to call a Data Logger A1or D44, etc. and have some location value.

Also, each Data Logger can have a user message or note up to 31 characters in length for site notes, etc.

The Integrated Water Level Data Logger System

Elevation. The elevation setting is the distance from the sensor to the reference zero or ground level. For example, for the ultrasonic sensor sitting at the top of the pipe, using an elevation of 42.67 inches for the top of the pipe, and an offset of 0 inches would give a reading of 0 inches for water down 42.67 inches. The elevation can be entered into a Data Logger from 0” to 655.35” to the nearest one-hundredth of an inch. For the pressure sensor, for example, sitting 50 inches below the ground surface or reference zero, enter a -50 for the elevation and 0 for the offset. A negative elevation denotes the sensor being below the ground surface and a positive elevation means the sensor is above the ground surface. Enter negative numbers on the HP by pressing the number followed by the +/- key.

The Hewlett-Packard 48GX Pocket Calculator/ Computer

The START MENU. All Data Logger functions are accessed using the pocket HP and the START MENU. The START MENU can be located by following the instructions on page 28, Starting the START MENU.

Memory. The pocket HP48GX stores as many data files that contain as many as 40,000 water level measurements. An optional PCMCIA card inserted in an HP slot, permits storage of 100,000 water level measurements. The data files downloaded from the Data Logger can be analyzed on screen by viewing or plotting data files, or printed to an optional infrared linked field printer. Data files are readily transferred to a PC with PC Transfer Software.

#### The Integrated Water Level Data Logger System

Batteries. Three AAA alkaline batteries power the HP48GX. A battery low annunciator, ((.)), at the top of the display indicates low battery status on the HP. A low battery message is displayed for three seconds,

Warning: LowBat ( ),

when the calculator is turned on during a low battery condition. Replace the HP batteries when a low battery condition is indicated. Follow the steps on the next page when replacing batteries. Continuing to use the HP in a low battery condition could result in loss of the START MENU library and data stored in the calculator.

Changing Batteries (from appendix A-5, HP 48 User’s Guide)

- Turn the calculator Off. Removing batteries with the HP On may result in program and data loss.

- Prepare three fresh AAA alkaline batteries to insert.

- Remove the calculator battery compartment door on the lower back of the HP by pressing down and sliding it off away from the calculator. Be careful not to press the calculator’s ON button.

- Remove the batteries and replace them immediately within two minutes to prevent memory loss.

- Follow battery polarity outlines on the bottom of the battery compartment.

- Slide the tabs on the battery compartment door into the slots in the calculator plastic case.

- Press the ON button to turn the calculator on.

## PC Transfer Software

The PC Transfer Software runs on a PC and allows the user to communicate with the Data Logger via a serial cable with a null modem adapter.  The software also supports transfer of data via serial cable from the Hewlett-Packard 48G+ calculator.

PC Transfer Software Installation

To install the PC Transfer Software, insert disk 1 into ‘A’ drive and select with the mouse Start>>Run, type A:\setup. Follow the setup instructions on your screen.

Once installed, click Start>>Programs, then click on PC Transfer 95 98 NT. A blank area with a menu along the top appears on your screen. The menu consists of File, Download, Set Data Logger, Handheld Retrieval Device, and About.

To communicate with the Data Logger, connect the serial cable to an unused Com port on the back of your PC. Connect the null-modem adapter end to the Data Logger. Now you are ready to communicate with your Data Logger.

From the menu, click on Download, then Current Status. An immediate measurement is taken and displayed on the screen along with the current time and date (see the section below to set current time and date), time and date of next measurement, time and date of memory wrap, name and serial number, offset in whole inches, battery condition and installation date, temperature, user notes, latitude-longitude coordinates, and elevation setting. The Current Status function gives the user information about the Data Logger without writing the information to the PC hard drive.

Click on Set Data Logger to see the options you can select and set up the Data Logger. The sections below describe the action of the Transmit button. The Transmit button sends the PC's current time and date, the time and date to begin taking measurements, the measurement interval, and the offset in inches.

The offset is the number of inches from the user desired Zero Mark to the Zero Mark on the pipe. The offset feature is useful when you want to effectively move the Zero Mark. If the Zero Mark on the pipe is 5 inches above the ground, setting an offset of 5 would yield a measurement of 0 inches for water at ground level.

An interval must be entered for information to be transferred to the Data Logger. However, a Time to Begin Logging is not absolutely necessary because the software will enter the next available time in the future. For instance, leaving zeros in Time to Begin Logging and using an interval of 1 hour, at 2:55 p.m. will set the Time to Begin Logging in the Data Logger to 3 p.m. If you would like the Data Logger to record measurements once a day at 2 p.m. beginning tomorrow, enter 1 Day, 14 Hours for Time to Begin Logging.

## PC Transfer Software

New Batteries. Clicking on New Batteries prompts the user if it is OK to write the current month/year to the Data Logger as the date of battery replacement. This feature lets the user know the batteries’ history.

Clear Data Logger.  Warning - Clears all data sets in memory and clears the Time to Begin to a date in the past. All DataLoggers are shipped with their data memories cleared.

Latitude-Longitude. Sends the Data Logger GPS coordinates of the format degrees, minutes.thousandths of a minute. This format can describe a coordinate to about 6 feet.

Name. The user can name a Data Logger with a name that begins with a letter and use up to 7 more letters or numbers. For example, A1, CD5, and FL455A are names that can be used.

The User Message can store field notes up to 31 characters.

Information about Elevation and Offset can be found on page 31.

Clicking on Download displays three types of downloads: Contiguous Dump, Dump Since Set, and Entire Memory.

Contiguous Dump downloads only that data which has been gathered since the user last downloaded the Data Logger using Contiguous Dump.

Dump Since Set downloads only that data which has been gathered since the Data Logger was last set with a measurement interval.

Entire Memory downloads all data in the Data Logger, including all data sets, up to 3906 measurements.

When downloading a Data Logger, a data file is written to the hard drive and stored in a directory named with the day’s date, like AUG0599. This organization of directories helps when  retrieving and archiving data files.

File>>Open displays data files which can be viewed and/or printed.

Clicking on the Handheld Retrieval Device enables the user to dump all data files collected with the Hewlett-Packard 48GX to the PC. They are processed and stored in a date named directory, e.g. Jan0297. Transferring files again the same day would create a directory named Jan0297.2 and so on.

The user must select the COM port to which the HP48 is connected, then click on Receive Files. Status Bars show the progress of the download. The HP48 shuts itself off when it is finished sending all of its files.

Loading the HP48 with the Start Library using Kermit

Note: Loading the Start Library can be tedious due to the multiple function keys present on the HP. Call our toll-free number, 1-888-808-5488, and we can save you time by walking you through the loading process.

To clear the entire HP48G+ memory of all directories, programs, libraries, and data, simultaneously press the ON-A-F combination of keys. When prompted "Try To Recover Memory?" select NO. The HP will beep and the screen will show "Memory Clear".

If there are other directories, programs, and data, on the HP48G+ that the user wishes to remain on the HP, go through the library removal process in this paragraph. When loading an updated Start Library on the HP48GX, if there is an existing Start Library on the HP, the Start Library being replaced must first be “Detached” and then “Purged”. Be sure to transfer all data before loading an updated Start Library. After all data has been transferred to a PC, the existing Start Library can be detached and purged. Change to the HOME directory by pressing the green right-shift key followed by the green HOME that is the third key down from the top left of the keyboard. To detach and purge a pre-existing Start Library on the HP, enter, without including the quotes, “:0:800 DETACH”, then “:0:800 PURGE”. The colon key is the green right-shift “+” key. Normally the 2 colons are placed on the bottom of the screen with a flashing insert arrow. Press the 0 key, followed by a right cursor key that is the third key down on the right of the HP. Press 800, followed by the SPC (space) key, that is the second key from the right on the bottom row. Now to enter the word DETACH by using the purple left-shift key followed by the 2 key (LIBRARY). Press the top right white key that corresponds to DETAC on the right bottom of the HP screen. With :0:800 DETACH on the screen, press ENTER. The procedure is similar for entering “:0:800 PURGE”. The only difference is to use purple left-shift key followed by the EEX key (PURG), that is the 3rd key from the left, 5 rows down. With :0:800 PURGE on the screen, press ENTER. Erase the existing INFINITIES directory on the HP by typing “‘INFINITIES’ PGDIR”. Use the single apostrophe (the third row down on the left), then enter the word INFINITIES by pressing the alpha key (the fourth key from the bottom on the left) and entering the individual letters. Use the SPC (space) key and the right cursor as necessary. Press the ENTER key. This purging takes about 30 seconds.

To install the Start Library on HP48GX units using a PC and HP48 cable, connect the 9-pin end of the cable to a COM port on the PC and the 4-pin end of the cable to the HP48GX. Insert disk 3 of the PC Transfer 95 software into ‘A’ drive and go to Windows Explorer clicking on Start, Programs, and Windows Explorer. Click on the A: icon. Double-click on Kermit.exe. At the Kermit-MS> prompt, set the port to which the cable is connected by typing “set port 1” (do not enter the quotation marks) for com1. Again at the Kermit-MS> prompt, type “set baud 9600” to set the communication speed. At the Kermit-MS> prompt

Loading the HP48 with the Start Library using Kermit

type “server”.  The PC is now ready to transfer the Start Library to the HP. On the HP, use green-shift HOME (3rd key down on the left) to switch to the HOME or root directory. Use purple-shift, I/O (the 1 key), then IOPAR (the 2nd white key from the left at the top) to ensure the settings are for wire transfer, binary transfer (toggle the white ASCII key for binary), 9600 baud, parity: none, checksum: 3, translate: 1. Once the settings are verified, press purple-shift, I/O again (the 1 key), then SRVR (top left white key). Type “{LIBFILE SETUP}”, not entering the quotation marks. Press the purple shift key followed by the “+” key with the purple braces on top (bottom left key). Use the alpha key for placing the HP in alphabet mode and enter the letters from the HP keyboard. Press ENTER. Now press KGET (3rd white key from left) and transfer begins. Be sure to have all other communication programs, such as Microsoft Exchange, closed or a conflict could result. Coms 1 and 3 share a common IRQ, as do Coms 2 and 4. Attempting to communicate on both Coms 1 and 3 at the same time can result in a conflict.

Progress of the transfer is shown on the Kermit window. The file name LIBFILE is transferred first and is about 38k in size. Kermit displays both the kilobytes transferred and the percent transferred. Transfer takes 4 to 5 minutes. End of transfer may be signaled by a beep. Then the small file SETUP is transferred in a second or two and another beep may be sounded signifying the end of transfer. The Kermit window shows Sending: Completed.

Press the VAR key (4th key from the left on the 2nd row) on the HP, which shows the user variables. Press SETUP (top left white key) and the HP readies itself to install the Start Library by turning itself off. Press and release the ON key and wait for the Start Library to be installed. The small hourglass in the upper-right corner signals the user that the HP is processing. When the hourglass disappears (in about ten seconds), press CST (3rd key from the left on the 2nd row) for the custom menu. Press STLIB (top left white key) for Start Library. Press START (top left white key) for the Start Menu. You can always access the Start Menu by getting the HP48’s attention by using the ON or CANCEL key followed by the CST key.

Exit the server mode in Kermit by closing the Kermit window or by using Ctrl-C. Then at the Kermit-MS> prompt type EXIT to exit Kermit. Now close the Kermit window.

Any questions on installing the Start Library can be directed to our toll-free line at 1-888-808-5488.

User Well Pipe Construction

Infinities USA Ultrasonic Water Level Data Loggers for on pipe use can measure water levels from 0 to 18 feet. Users can construct their own well pipe, which provides the flexibility to use the Data Logger in various applications.

Well Pipe Construction Materials

- A 2" or 3” PVC sch, 40 pipe. (3” DWV type pipe is less expensive and lighter than pressure type 3” pipe.)

- A well point with 0.008" or 0.010" screen with a coupler.

- PVC cement.

Well Pipe Construction Steps

- The Ultrasonic Water Level Data Loggers have a minimum distance they can sense of about 16"-18". Placing the Zero Mark 24 inches down from the top of the pipe will allow for 6"-8" of water level measurement above the Zero Mark. Therefore, cut the length of pipe 24" + the desired range below the Zero Mark. For example, for a 40-inch range below the Zero Mark, cut the PVC pipe 64". Clean any PVC shavings from the inside of the pipe.

- Apply PVC cement to the outside of the bottom of the pipe and to the inside of the well point coupler. Couple the point to the pipe, wiping away any excess cement.

- Measure 24 inches down from the top of the pipe to place your Zero Mark label.

In the above example, the water level Data Logger will log water levels from the Zero Mark to the bottom coupler as 0 to –40 inches. Water levels above the Zero Mark will read positive from 0 to ~ + 6 to +8 inches. If a larger positive range is needed, use the offset feature and/or the elevation feature of the Data Logger. Offsets up to 255 inches (no fractions) can be used. Offsets effectively move the Zero Mark down the pipe. The Elevation feature can also be used to move the Zero Mark to any place on the pipe.

The Pressure Water Level Data Logger sensor can sit on the bottom of a well or be suspended. The sensor can also sit on a 1-inch bed of small gravel in the bottom of the well. The Elevation feature can be used to tell the data logger where the sensor is relative to the ground surface or zero reference. For example, a sensor 50 inches below the ground surface would be given a -50 inch elevation. Using the HP, use the Elevation feature to enter a -50 by entering 50 and then using the +/- key.

Rain Gauge Data Logger with Self-Emptying Tipping Bucket

The Rain Gauge Data Logger and the Water Level Data Logger have similar logging operations with PC Transfer Software and the HP48GX.

Installation

Choose an open site where the rainfall does not get blocked by the canopy.

To install the Rain Gauge Data Logger, auger a hole, two to three feet deep, to support the 3-inch pipe, which is five feet long. Insert the pipe and backfill.

Press the Rain Gauge assembly on the pipe and seat the coupler. Level the bucket as necessary. Remove the shipping twist tie from the tipping mechanism.

Insert the screen in the collector to catch any leaves or straw that may blow into the collector.

Setup

To log daily rainfall, set the Data Logger to log the accumulation at 11:59:59 p.m. (enter 23:59:59 for Time to Begin Reading) with the Time Interval Between Readings set to 24 hours. Setting the time one second short of the next day keeps the rainfall accumulation logged on the correct day.

When installing the rain gauge, be sure to run “Zero Rain Gauge”. This clears any value that may be in  “Amt. to log:”. Zero Rain Gauge is at the bottom of the Start Menu on the HP. Zero Rain Gauge is on the Set Data Logger window in the PC Transfer Software.

There is no temperature data or offset information in the rain gauge data file.

Note: Be sure to remove the shipping twist tie from the tipping bucket mechanism.

Pressure Water Level Data Logger

The Pressure Water Level Data Logger and the Ultrasonic Water Level Data Logger have similar logging operations with PC Transfer Software and the HP48GX.

Installation

The Pressure Water Level Data Logger fits down 2" diameter wells.

Setup

The Elevation feature can be set to one-hundredth of an inch and can be a positive or negative number. The Elevation feature produces water level data in reference to the Zero-Mark. For example, if the ground surface is the reference Zero-Mark and the pressure sensor is 80 inches below the ground surface, a -80 inch Elevation would produce water level data of 0 inches when the water is at the ground surface. Furthermore the water level data would be -80 inches when the water level is at the bottom of or below the pressure sensor.

The atmospheric vent tube provides barometric pressure compensation. The vent tube is kept dry with the desiccant. Renew the desiccant when the desiccant turns from blue to pink. Renew by removing the desiccant from the tube, place in a bowl, and microwave for 5 to 6 minutes.

There is no temperature data in the pressure water level data file.

Specifications for Pressure Water Level Data Loggers

Number of measurements in memory: 3,906

User programmable interval: 1-reading/second to 1-reading/6 months

User interface to Logger: PC or opt. HP 48GX or 48G+ Calculator w/ software

Data Logger power: Four AA alkaline batteries

Data Logger battery life, typical: 4 years

Data Logger range: 0 to 11.5 ft, 0 to 34.5 ft, 0 to 69 feet, 0 to 115 ft, 0 to 230 ft

Ranging environment: inside 2” diameter or larger pipe

Temperature compensation: 0 °F to 120 °F

Humidity: to 100%

Accuracy: +/- 0.1% of pressure sensor range

Resolution: 0.01 inches

Data Download rate: 150 measurements per second

Download medium: serial cable

Hewlett-Packard 48GX storage: multiple Data Loggers, 40,000 measurements

Specifications for Ultrasonic Water Level Data Loggers

Number of measurements in memory: 3,906

User programmable interval: 1-reading/second to 1-reading/6 months

User interface to Logger: PC or opt. HP 48GX or 48G+ Calculator w/ software

Data Logger power: Four AA alkaline batteries

Data Logger battery life, typical: 4 years

Data Logger range: Model #220, 18 feet; Model #420, 35 feet.

Minimum target distance: ~16 inches

Ranging environment: Model #220, inside 1 ¼” to 3” PVC sch. 40 pipe

Model #420, above surface water, without pipe

Temperature compensation: 0 °F to 120 °F

Humidity: to 100%

Accuracy: +/- 1% of distance measured

Resolution: ~0.04 inches or 1mm

Data Download rate: 150 measurements per second

Download medium: serial cable

Hewlett-Packard 48GX storage: multiple Data Loggers, 40,000 measurements

##### Specifications for Rain Gauge Data Loggers

Number of measurements in memory: 3,906

User programmable interval: 1-reading/second to 1-reading/6 months

User interface to Logger: PC or opt. HP 48GX or 48G+ Calculator w/ software

Data Logger power: Four AA alkaline batteries

Data Logger battery life, typical: 4 years

Operating Temperature of Data Logger: 0 °F to 120 °F

Operating Temperature of non-heated Tipping Bucket: 32 °F to 120 °F

Humidity: to 100%

Accuracy: +/- 1%

Resolution: 0.01 inches

Data Download rate: 150 measurements per second

Download medium: serial cable

Hewlett-Packard 48GX storage: multiple Data Loggers, 40,000 measurements

Rain Collector Dimensions: 6.5” diameter x 9.5” high

##### Specifications for Soil and Water Temperature Data Loggers

Number of measurements in memory: 3,906

User programmable interval: 1-reading/second to 1-reading/6 months

User interface to Logger: PC or opt. HP 48GX or 48G+ Calculator w/ software

Data Logger power: Four AA alkaline batteries

Data Logger battery life, typical: 4 years

Temperature Range: 0 F to 140 F

Accuracy: +/- 0.2 F

Resolution: 0.1 F